SOLANO COMMUNITY COLLEGE

MT 162, ROBOTICS

Fall, 2018

LAB 1:  INTRO LAB

Catharine Crayne

9/18/18

OBJECTIVE:

- To learn how to perform basic actions of the robot and learn the menu structure on the teach pendant.

GROUP MEMBERS

N/A

MATERIALS:

RoboGuide and Laptop, or Fanuc Robot

PROCEDURE:

- Power up the robot controller.  Place the robot in Teach Mode.

- Turn on Teach Pendant and change the robot’s jog speed to 50%.  Set the robot to jog in the JOINT coordinate system.

- What is the fastest speed available to jog the robot?

- What is the slowest speed?

- What increments does the speed change by when the [+%] key is pressed?

- What increments does the speed change by when the SHIFT key is pressed with the [+%] key?

- Jog all 6 Axis’ of the robot.

- Now set the robot to jog in the WORLD coordinate system.

- Jog the robot in the WORLD frame so that you keep the point or tooltip perpendicular with each side of a box.

- Press [MENU] key and take note to what the top of the pop up menu screen says.

- Press [FCTN] key then select 0 for Next.  Select Quick/Full Menu and press [ENTER].

- Now press [MENU] key.  What is changed on the menu?

- Return the [MENU] to its full menu following step 11.

- Jog the robot to align each axis witness mark using JOINT jogging.  Change the speed to FINE or VFINE if needed.

- Use the [POSN] key and record the values of each joint.

- Using the MENU, go to SYSTEM, TYPE, and AXIS LIMITS.

- Write down the axis limits for each joint.

- Reduce the axis limit for axis 1 by 20 degrees in each direction.  Restart the controller.

- Jog joint 1 so that it generates a motion limit error.  Enter into the MENU system and go to ALARM.  View the Alarm History (HIST).  Write down the error.

- Press [SHIFT] and [DIAG] to display Cause and Remedy for that error.

- Return the axis limit back to it original value.  Restart the controller.

- What is the Software Edition Number for your controller?

- What is the Software Serial Number?

- What is the F Number of the robot?

RESULTS - DATA

Step 3: 100%

Step 4: VFine

Step 5: 5%

Step 6: 50%

Step 10/12: Less options are available on the Quick Menu. The menu is called “Quick 1”.

Step 15:

Step 17:

Step 19: MOTN-017 Limit error (G:1, A2,3)

Step 22: V9.10P/08

Step 23: 144786

Step 24: F217912

OBSERVATIONS

Step 2				       			 Step 3

Step 4							Step 10

Step 11/12

PROBLEMS/QUESTIONS:  ANSWER THESE!

- What key do you press to change coordinates when trying to jog the robot?

Shift + (-X or +X or -Y or +Y or -Z or +Z)/J1-J6

- What direction does the robot move in the WORLD coordinate system for Z+ (floor mounted)

Up (vertically)

- How can you access or find out the current coordinate values of the robots position?

Positional data can be select by the MENU key or the POSN key.

- When a DCS or Motion Error occurs, what two keys must be pressed in order to clear the error?

Shift + Reset

- Name two ways to control the robots motion limits or boundaries.

Axis Limit software settings (raise (+) and lower (-) motion degree limitations) and Axis limit switches.

- How can a singularity error be corrected?

Place the controller in Joint mode and jog 5-10 degrees out of the singularity.