SOLANO COMMUNITY COLLEGE

MT 162, ROBOTICS

Fall, 2018

SOLANO COMMUNITY COLLEGE

ROBOTIC SYSTEMS MT-162

LAB 12:  Macros

Catharine Crayne

11/16/18

OBJECTIVE:

- To learn how to create a macro program and set it up to run from Teach Pendant, Digital Input, and the Manual Functions Screen.

GROUP MEMBERS

Dianne Halsey, Jerry Livingston, Alex

MATERIALS:	RoboGuide and Laptop, Fanuc Robot or RoboGuide

PROCEDURE:

- Power up the robot controller.  Place the robot and pendant in Teach Mode.

- Create a new program that will be called ZERO_POS.

- Teach the robot to go to position register (5) where all joints are at 0 degrees.

- Verify that it works.

- Change the program to a Macro sub type and assign it to the SU key #1.

- Verify that it works with the SU key.

- Assign a Macro sub type to one of the earlier programs you wrote, such as Triangle.  Assign it to the Manual Functions (MF) screen #4.

- Now assign a Macro sub type to another on of your earlier programs such as Box.  Assign it to DI [109].

RESULTS - DATA

Not Applicable for this lab.

OBSERVATIONS

Step 2							Step 3

Step 5-6						Step 7

Step 8

Step 2	- 8

/PROG  LAB12_ZERO_POS_JERDIACATALE      Macro

/ATTR

OWNER        = MNEDITOR;

COMMENT        = "";

PROG_SIZE    = 150;

CREATE        = DATE 18-11-13  TIME 20:18:44;

MODIFIED    = DATE 18-11-13  TIME 20:32:40;

FILE_NAME    = ;

VERSION        = 0;

LINE_COUNT    = 1;

MEMORY_SIZE    = 518;

PROTECT        = READ_WRITE;

TCD:  STACK_SIZE    = 0,

TASK_PRIORITY    = 50,

TIME_SLICE    = 0,

BUSY_LAMP_OFF    = 0,

ABORT_REQUEST    = 0,

PAUSE_REQUEST    = 0;

DEFAULT_GROUP    = 1,*,*,*,*;

CONTROL_CODE    = 00000000 00000000;

/MN

1:J PR[5] 10% FINE    ;

/POS

/END

Analysis Questions:

- What are five ways to execute an assigned Macro?

In Manual Mode you can use the Teach Pendant User Keys (UK/SU) to start the macro. It can also be executed via the Manual Functions Screen (MF).

In Automatic Mode, you can execute the macro through the Digital and Robot Inputs (DI/RI). Another option to execute the Macro in Automatic Mode is through the UOP Inputs (UI).

Macros can also be executed during the program execution, such as when the Macro instruction is listed within the program (is the actual instruction). Alternatively, the program can place a CALL instruction to the Macro from within the program.

- Why can’t your ZERO_POS macro (from step 5) be assigned to User Key 1 (UK[1])?

Because the ZERO_POS macro was programmed as SU (Shift User) and it would need to be programmed as UK in order to be assigned to UK[1].