SOLANO COMMUNITY COLLEGE

MT 162, ROBOTICS

Fall, 2018

SOLANO COMMUNITY COLLEGE

ROBOTIC SYSTEMS MT-162

LAB 5:  MOTION INSTRUCTION PROGRAMMING

Catharine Crayne

10/22/18

OBJECTIVE:

- To learn how to create, teach, test, and execute a motion program that use various types of instructions.

Group Members

Diane

MATERIALS:	RoboGuide and Laptop, Fanuc Robot, Box

PROCEDURE:

- Power up the robot controller.  Place the robot in Teach Mode.

- Turn on Teach Pendant and change the robot’s jog speed to 25%.  Set the robot to jog in the TOOL coordinate system.

- Set the active Frames to Tool 2 and User 0.  Remember [SHIFT] + [COORD].

- Verify Tool Frame 2 is taught correctly by Jogging the robot in TOOL, yaW, Pitch, and Roll.  If it is not correct, re-teach it.

- Set the robot to jog in JOINT coordinates.

- Create 2 new Position Registers, HOME and SAFE.

- Jog the robot to a HOME joint position (or close to it).  Use the [DATA] key and record the position into a PR.

- Give the PR a comment “HOME”.

- Now jog the robot to a SAFE position off to the side.  Record this position into another P and comment it with “SAFE”.

- Create a new program, “BoxLab5YourName”.

- Include the USER Frame and TOOL Frame info in the first couple of lines.

- Have the program go to the HOME PR position with a Joint move.

- Then teach another simple path that goes from the HOME PR….. outlines a box shape on the table, then returns back to HOME.  See example to the right..

- Test your program in STEP mode.

- Now test the program in continuous mode.

- Modify the path of the motion to include another small square as shown using L P[ ] 200mm/sec CNT 50 instructions.

- Test your program in STEP mode.

- Now test the program in continuous mode.

- Now turn off the Teach Pendant, set the Operator Panel to AUTO, and run your program in AUTO mode.

RESULTS - DATA

No Procedure questions were included in this lab.

OBSERVATIONS

Step 9							Step 10

Step 11				  Step 11

Step 12-16

Analysis Questions:

- What combination of keys are used to record a position?

[SHIFT] + [Record]

- How is FINE termination type differ from CNT termination type?

FINE stops at each point, and Continuous (CNT) keeps going in a fluid motion.

- What status symbol is used to indicate that the robot is currently at the taught position?

@

- How is Position Register data different than just Position data?

Position Register data is a global position which can be used across programs, whereas Position data is a position point which can only be called/used in the program in which it was created.