SOLANO COMMUNITY COLLEGE

MT 162, ROBOTICS

Fall, 2018

SOLANO COMMUNITY COLLEGE

ROBOTIC SYSTEMS MT-162

LAB 6:  MOTION INSTRUCTION PROGRAMMING PT. 2

Catharine Crayne

10/22/18

OBJECTIVE:

- To learn how to create, teach, test, and execute a motion program that use various types of instructions and creates a variety of motion shapes.

Group Members

N/A

MATERIALS:	RoboGuide and Laptop, Fanuc Robot, Box

PROCEDURE:

- Power up the robot controller.  Place the robot and pendant in Teach Mode.

- Set the active Frames to Tool 2 and User 0.  Remember [SHIFT] + [COORD].

- Verify Tool Frame 2 is taught correctly by Jogging the robot in TOOL, yaW, Pitch, and Roll.  If it is not correct, re-teach it.

- Set the robot to jog in JOINT coordinates.

- Make sure the robot has a Position Registers for HOME.

- Create a new program, “SHAPESLab6YourName”.

- Include the USER Frame and TOOL Frame info in the first couple of lines.

- Have the program go to the HOME PR position with a Joint move.

- Then program the robot to trace a square, triangle, and circle, returning back to HOME in between each.  See example to the right..

- Test your program in STEP mode.

- Now test the program in continuous mode.

- Now turn off the Teach Pendant, set the Operator Panel to AUTO, and run your program in AUTO mode.

- Now Copy the Program and name it “SHAPESRLab6YourName”.

- Edit this program and use the copy and paste functions to reverse the motion of the program.  Test and verify its operation.

RESULTS - DATA

No Procedure questions were included in this lab.

OBSERVATIONS

Step 2								Step 8

Step 9							        Step 9

Step 13					Step 14			Step 14

Analysis Questions:

- What could you do to look at the positional data of P[3] in your program?  Describe the steps.

- What could you do to run the same program 20 mm above the table surface?

- How is R-POSID different from RM-POSID?

R-POSID adds the lines in reverse order and retains their original position numbers (works best when pasting inside of the same program).

RM-POSID keys do not paste instructions exactly as written. Motion instructions are reconstructed so that motion, speed and termination occur at the proper places in a path.