SOLANO COMMUNITY COLLEGE

MT 162, ROBOTICS

Fall, 2018

SOLANO COMMUNITY COLLEGE

ROBOTIC SYSTEMS MT-162

LAB 9:  INPUTS / OUTPUTS

Catharine Crayne

11/7/18

OBJECTIVE:

- To learn how to configure and utilize I/O in a program.

GROUP MEMBERS

Joel

MATERIALS:	RoboGuide and Laptop, Fanuc Robot w/gripper

PROCEDURE:

- Power up the robot controller.  Place the robot and pendant in Teach Mode.

- Set up Robot Outputs 3 and 4 as complementary signals.  To do this, press [I/O] and select [TYPE] and select Robot.  To switch between inputs and outputs, press the [IN/OUT] softkey.

- How many Robot Inputs and Outputs are available to use on this controller?

- Scroll down to Robot Output 3.  Select the [DETAIL] softkey to open up the Details screen.  Set the Complementary Pair option to TRUE.

- Now exit this Details screen by selecting [PREV] and then open up the Details screen for Robot Output 4.

- What does it say in the Complementary Pair option?

- Verify RO[3] and RO[4] work as complementary outputs.

- Exit this Details screen.  Select [TYPE] and select Digital. Then select the [CONFIG] softkey to open up the configuration menu.

- What are the Configuration settings for Digital Inputs (Rack, Slot, Starting Points)?

- What are the Configuration settings for Digital Outputs (Rack, Slot, Starting Points)?

- Exit out of the configuration screen, switch to digital inputs, and choose the [MONITOR] softkey.  Which SOP switch (not the MAIN POWER) is connected to DI[8]?

- Safety devices are wired to DI[89-98].  Which safety switch is wired to DI[93]?  DI[95]?

- What Rack and Slot are the above devices configured to?

- The area scanner is wired to DI[109] and DI[111].  How are their inputs different?

- Switch Digital Outputs.  Scroll down and activate DO[101].  What does it do?

- Activate DO[105-108].  What do they do?

- Create a short program that opens the gripper, waits 1 second, then closes the gripper, waits 1 second, and repeats this in a loop.

- BONUS.  Create a program that CALLS the above gripper program (without the endless loop)  if someone walks too close to the area scanner, and then stops if they walk away.

RESULTS - DATA

Step 3: 8 inputs & 8 outputs

Step 6: There isn’t one, so nothing.

Step 9:  Digital Input Configuration Settings

| Range | Rack | Slot | Starting Point |
| --- | --- | --- | --- |
| DI[1-9] | 34 | 1 | 21 |
| DI[10-80] | 0 | 0 | 0 |
| DI[81-88] | 48 | 1 | 21 |
| DI[89-90] | 36 | 7 | 1 |
| DI[91-98] | 36 | 3 | 1 |
| DI[99-100] | 0 | 0 | 0 |
| DI[101-120] | 48 | 1 | 1 |
| DI[121-512] | 0 | 0 | 0 |

Step 10: Digital Output Configuration Settings

| Range | Rack | Slot | Starting Point |
| --- | --- | --- | --- |
| DI[1-8] | 34 | 1 | 1 |
| DI[9-19] | 34 | 1 | 21 |
| DI[20-80] | 0 | 0 | 0 |
| DI[81-84] | 48 | 1 | 21 |
| DI[85-100] | 0 | 0 | 0 |
| DI[101-120] | 48 | 1 | 1 |
| DI[121-512] | 0 | 0 | 0 |

Step 11: ON/OFF switch for Teach Pendant

Step 12: DI[93] is wired to E-Stop; DI[95] is wired to the Deadman’s switch

Step 13: Rack 36, Slot 3 & 7

Step 14: They are proximity sensors, and one of the sensors has a longer scanning range (slows down robot), whereas the input within the smaller range stops the robot.

Step 15: Laser

Step 16: DO[105] = Horn; DO[106] = Green Light; DO[107] = Amber Light; DO[108] = Red Light

OBSERVATIONS

Step 2	- 7

Step 9 							Step 10

Step 14

Step 17

Analysis Questions:

- What are the various types of I/O signals?

Robot Output Signals: Gripper; Vacuum On; Vacuum Off.

I/O Digital Out: IMSTP; *HOLD; *SFSPD; CYCLE START; FAULT RESET; Start; Enable; Laser; Light; Horn; Green Light; Amber Light; Red Light; Small Suction.

I/O Digital In: CSC1; CSC2; SVOFF SS; FENCE SS; EXEMG SS; NTED SSI; OPCEMG SSI; AUTO SSI; T1 SSI; T2 SSI; Hold Zone; Rst Sig; Slow Zone.

- Why might it be necessary to simulate a digital or robot input?

To test the inputs, outputs, parts or programs prior to putting the robot into production.

- Group I/O is useful for communicating binary information.  If Digital Inputs 1 - 4 are configured as GI[1], then what is the value of GI[1] when DI[1] = 1, DI[2] = 0, DI[3] = 0, and DI[4] = 1?

8+0+0+1 = 9

GI[1] = 9